F. Y. B. Tech Academic Year 2020-21

Subject: Basics of Electrical and Electronics Engineering     Trimester: I

Name: Shreerang Mhatre                                        		                   Division: 11

Roll No: 111056          		        		                                           Batch: K3

Experiment No:7

Name of the Experiment:  Verification of Kirchhoff’s current law (KCL) and Kirchhoff’s voltage law (KVL)

Performed on:  04/02/2022

Submitted on:  11/02/2022

Aim:  Verification of Kirchhoff’s current law (KCL) and Kirchhoff’s voltage law (KVL)

Objectives

- To get the idea of analysing the circuits

- To solve the electrical circuit theoretically

- To verify practical and theoretical value

Components and equipment required

| Components | Specification |
| --- | --- |
| DC Ammeters | 0 – 1 A (3 No.) |
| DC Voltmeter | 0 - 20 V (1 No.) |
| Circuit board | Same as shown in Fig.3 |
| DC Power supply | 0 – 15 V (2 No.) |

Theory

A German physicist Gustav Kirchhoff developed two laws enabling easy analysis of an interconnection of any number of circuit elements. The first law deals with the flow of current and is popularly known as Kirchhoff’s Current Law (KCL) while the second one deals with the voltage drop in a closed network and is known as Kirchhoff’s Voltage Law (KVL).

- Kirchhoff’s current law

It states that, at any moment, the algebraic sum of currents flowing through a point (or junction) in a network is Zero or in any electrical network, the algebraic sum of the currents meeting at a point (or junction) is Zero. This law is also known as Point Law. In other words, the sum of the currents flowing towards a point is equal to the sum of those flowing away from it.

Fig. 1: Kirchoff’s current law

In Fig.1, currents I1, I4 and I5 are flowing towards the node, are considered as positive. At the same time, currents I2 and I3 are leaving the junction, are considered as negative. As per KCL, sum of all the currents is zero. That means sum of currents entering the node and sum of currents leaving the node is equal.

- Kirchhoff’s voltage law

It states that, in any closed loop (mesh), the algebraic sum of the EMFs applied is equal to the algebraic sum of the voltage drops in the elements. It is also known as Mesh law.

Fig. 2: Kirchoff’s voltage law

Component Values used:

Voltage source E=

R1=                   , R2=                 R3=

Circuit diagram

Components used in the circuit

V1 = _____V

V2 = _____V

R1 = ________Ω

R2 = _____Ω

R3 = _____Ω

Procedure:

A. KCL

1) Connect the circuit as shown in the diagram

2) Measure currents in every branch and note it in observation table as practical value.

3) Calculate theoretical current values using mesh equations

- KVL

1) Connect the circuit as shown in the diagram

2) Using a voltmeter, measure the source voltage E and the voltage drop across the resistances R1, R2 and R3.

3) Find the current  I= E / R1+R2+R3

4) Calculate theoretical values as V1=IR1, V2=IR2, V3=IR3

Observation Tables:

- Verification of KCL-

| Current (A) | Theoretical Value | Practical value |
| --- | --- | --- |
| I1 | 16.448mA | 16.4mA |
| I2 | 2.889mA | 2.89mA |
| I3 | 19.337mA | 19.3mA |

Calculate I1+I2  and compare it with I3. These two values should be equal.

- Verification of KVL-

| Voltage (V) | Theoretical Value | Practical value |
| --- | --- | --- |
| VR1 | 1.98V | 1.98V |
| VR2 | 5.04V | 5.05V |
| VR3 | 2.97V | 2.97V |

VR1 ,VR2and VR3 are the voltage drops across resistances R1 R2 and R3. Calculate  VR1+VR2+VR3and compare it with E. These two values should be equal.

Conclusion:

Hence, we verified Kirchhoff's Voltage Law (KVL) and Kirchhoff's Current Law (KCL) by simulating circuits on Tinker cad and matched theoretical values with our

values from Tinker Cad.

Calculations:

Kirchhoff’s current law Circuit:

Kirchhoff’s voltage law Circuit:

Post-Lab Questions

- Theoretically calculate the voltages and currents for each element in the circuit and compare them to the measured values.

- Calculate the percentage error in the two measurements and provide a brief explanation for the error.

Note: Students are instructed to do all necessary calculations and answer the questions on separate sheets and attach them.